IT-SIMPLERA INSTITUTE Cyber Security Analyst Internship Program WEEK 02 ASSIGNMENT REPORT Computer Networking Fundamentals & Network Configuration Prepared By Abdullah Bachelor of Information Technology, University of Gujrat Assigned To Talal Rehan — Senior Cyber Security Analyst (Supervisor) Submission Deadline: 10 July 2026 Registration No: REDB01-1815

# Table of Contents

1. Introduction

2. Task 1 — Network Information (ipconfig /all)

3. Task 2 — Basic Networking Commands

4. Task 3 — Router vs. Switch

5. Task 4 — IPv4 vs. IPv6

6. Task 5 — The OSI Model

7. Network Diagram

8. Challenges Faced

9. Conclusion

10. References

# 1. Introduction

This report has been prepared as part of the Week 02 Assignment under the Cyber Security Analyst Internship Program at IT-SIMPLERA Institute, supervised by Talal Rehan. The assignment focuses on the foundational concepts of computer networking, including network configuration analysis, essential command-line diagnostic tools, the distinction between routers and switches, a comparison of IPv4 and IPv6 addressing, and the seven-layer OSI reference model.

The objective of this exercise is to build practical, hands-on familiarity with networking commands while reinforcing the theoretical framework that underpins network communication — knowledge that forms a critical foundation for further study in cybersecurity and network administration.

# 2. Task 1 — Network Information (ipconfig /all)

The ipconfig /all command was executed on a Windows host to retrieve detailed network configuration information. The key parameters identified are explained below:

| Parameter | Purpose |
| --- | --- |
| IPv4 Address | The unique logical address assigned to the device on the local network, used to identify and route traffic to and from the host. |
| MAC Address | The physical hardware address burned into the network interface card (NIC); used for communication at the Data Link layer within a local network segment. |
| Default Gateway | The IP address of the router that forwards traffic from the local network to external networks, such as the Internet. |

Figure 1: ipconfig /all output — IPv4 Address, MAC Address, and Default Gateway highlighted

# 3. Task 2 — Basic Networking Commands

The following essential networking commands were executed to diagnose and analyze network connectivity. Each command's purpose is summarized below, followed by a placeholder for the corresponding output screenshot.

| Command | Purpose |
| --- | --- |
| ipconfig /all | Displays complete IP configuration for all network adapters, including IPv4/IPv6 address, subnet mask, default gateway, DNS servers, and MAC address. |
| ping google.com | Tests reachability of a host by sending ICMP echo requests and measuring round-trip time, confirming basic connectivity. |
| tracert google.com | Traces the route packets take to reach a destination, listing each intermediate hop (router) and its response time. |
| nslookup google.com | Queries DNS servers to resolve a domain name into its corresponding IP address, useful for diagnosing DNS issues. |
| arp -a | Displays the ARP cache, mapping known IPv4 addresses to their corresponding MAC addresses on the local network. |

Output: ipconfig /all

Figure 2: ipconfig /all — full adapter configuration

Output: ping google.com

Figure 3: ping google.com — connectivity test with round-trip times

Output: tracert google.com

Figure 4: tracert google.com — hop-by-hop route trace

Output: nslookup google.com

Figure 5: nslookup google.com — DNS resolution result

Output: arp -a

Figure 6: arp -a — local ARP cache mapping IP to MAC addresses

# 4. Task 3 — Router vs. Switch

Routers and switches serve distinct roles within a network. The comparison below outlines their key differences:

| Comparison Point | Router | Switch |
| --- | --- | --- |
| Primary Function | Connects and routes traffic between different networks | Connects devices within the same local network (LAN) |
| OSI Layer | Network Layer (Layer 3) | Data Link Layer (Layer 2) |
| Addressing Used | IP Address | MAC Address |
| Traffic Handling | Routes data packets between networks using routing tables | Forwards data frames between devices using a MAC address table |
| Broadcast Domain | Separates broadcast domains | Operates within a single broadcast domain (unless VLANs are used) |
| Common Use Case | Connecting a home/office LAN to the Internet (WAN) | Connecting computers, printers, and servers within an office LAN |

# 5. Task 4 — IPv4 vs. IPv6

IPv4 and IPv6 are the two versions of the Internet Protocol used for addressing devices on a network. Their key differences are summarized below:

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address Length | 32-bit | 128-bit |
| Address Format | Dotted decimal (e.g., four numbers separated by dots) | Hexadecimal, colon-separated (eight groups) |
| Total Addresses | Approximately 4.3 billion | Approximately 340 undecillion |
| Address Configuration | Manual or DHCP | Supports auto-configuration (SLAAC) and DHCPv6 |
| Header Complexity | More complex, includes checksum | Simplified, fixed-length header for faster processing |
| Security | Security (IPsec) is optional | IPsec support is built-in |
| NAT Requirement | Commonly requires NAT due to address shortage | Designed to eliminate the need for NAT |

Example Addresses:

- IPv4 Example:  192.168.1.1

- IPv6 Example:  2001:0db8:85a3:0000:0000:8a2e:0370:7334

# 6. Task 5 — The OSI Model

The Open Systems Interconnection (OSI) model divides network communication into seven distinct layers, each with a specific function, associated protocol, and networking device:

| Layer # | Layer Name | Primary Function | Common Protocol | Associated Device |
| --- | --- | --- | --- | --- |
| 7 | Application | Provides network services directly to end-user applications | HTTP, FTP, DNS | Firewall / Gateway |
| 6 | Presentation | Translates, encrypts, and compresses data for the application layer | SSL/TLS, JPEG | Gateway |
| 5 | Session | Establishes, manages, and terminates sessions between applications | NetBIOS, RPC | Gateway |
| 4 | Transport | Ensures reliable end-to-end data delivery and error recovery | TCP, UDP | Firewall |
| 3 | Network | Handles logical addressing and routing between networks | IP, ICMP | Router |
| 2 | Data Link | Manages physical addressing (MAC) and error detection within a local segment | Ethernet, ARP | Switch, Bridge |
| 1 | Physical | Transmits raw bit streams over a physical medium | Ethernet (cabling), USB | Hub, Cables, NIC |

# 7. Network Diagram

The diagram below illustrates a sample network topology showing how end devices, a switch, a router, and the Internet are interconnected — consistent with the concepts explored in this assignment.

Figure 7: Sample network topology — end devices, switch, router, and Internet

# 8. Challenges Faced

- Interpreting the detailed output of ipconfig /all required cross-referencing each field with its theoretical definition to ensure accurate explanations.

- Differentiating OSI layer responsibilities for the Session and Presentation layers required additional research, as their functions can overlap in practice with modern protocol stacks.

- Formatting comparison tables to be both concise and comprehensive across six or more points needed careful selection of the most relevant distinguishing factors.

# 9. Conclusion

This assignment provided a comprehensive, hands-on introduction to core networking fundamentals essential to a cybersecurity analyst's skill set. Through direct use of diagnostic commands and structured comparisons of key networking concepts, a solid foundation has been established for further exploration of network security, traffic analysis, and infrastructure defense in subsequent weeks of the internship program.

# 10. References

- Microsoft Learn — Windows Networking Command Reference (ipconfig, ping, tracert, nslookup, arp)

- Cisco Networking Academy — Introduction to Networks: Routers and Switches

- IETF RFC 791 — Internet Protocol (IPv4) Specification

- IETF RFC 8200 — Internet Protocol, Version 6 (IPv6) Specification

- ISO/IEC 7498-1 — OSI Reference Model